ARCUS

Path 01 - Project-Context Design Recommendations

Implementation report for strengthening New Territory outputs as a professional preliminary design-support briefing

| Decision area | Recommended action |
| --- | --- |
| Path 01 scope | Keep province-only selection for the current release. Remove the custom drawing option from this workflow. |
| Report value | Use project context + dominant driver to generate preliminary design-support recommendations. |
| Current Torino case | Province: Torino; Project context: Bridge; Dominant driver: Hydraulic; Evidence: 42 events and 125 linked sources. |
| Mandatory PDF fixes | Remove browser print artifacts and replace the schematic map with a real ARCUS/Leaflet capture. |

# 1. Executive decision

The report content is now largely complete. The main missing value is not another generic analytical section, but a stronger connection between the user-selected project context and the dominant territorial driver identified by ARCUS.

For Path 01, the clearest product strategy is to keep province-only selection. This makes the report stable, avoids ambiguity between province-level and site-specific analysis, and supports a clear preliminary screening positioning.

| Keep | Remove / avoid |
| --- | --- |
| Province selection as the only Path 01 spatial input. | Custom polygon drawing in Path 01. |
| Province-level wording: selected province, administrative boundary, province-level screening. | Mixed wording such as selected area / custom geometry when the report is province-based. |
| Project-context recommendations based on Bridge, Road crossing, Railway crossing or Urban infrastructure. | Generic recommendations that ignore the user-selected project context. |

# 2. Problems still visible in the current PDF

| Issue | Why it matters | Action |
| --- | --- | --- |
| Map is still schematic | The report says the map is a real capture, but visually it is still a preview. This reduces trust. | Insert a true Leaflet/ARCUS map image with basemap, provincial boundary, events and active layers. |
| Browser artifacts still visible | Date/time, page title and about:blank make the PDF look like a browser print. | Generate PDF using Puppeteer/Playwright with displayHeaderFooter disabled. |
| Executive Summary wording | The sentence contains a repetition: high attention infrastructure attention level. | Use: The selected province requires high attention for new bridge interventions and territorial planning. |
| Project context underused | The user selected Bridge, but the report recommendations remain broadly generic. | Add Project-Context Design Focus and generate recommendations from project context + dominant driver. |

# 3. What ARCUS can claim safely

| Can state | Must not state |
| --- | --- |
| The selected province shows a hydraulic-dominated historical precedent pattern. | The planned bridge is unsafe. |
| Preliminary design discussions should prioritise hydraulic compatibility and scour-related checks. | Foundations must be designed in a specific way. |
| The report supports screening, due diligence and investigation planning. | The report replaces inspections, hydraulic modelling, geotechnical studies or structural design checks. |
| Comparable historical collapses can guide follow-up questions and priorities. | Comparable historical collapses predict future failure of a specific asset. |

Positioning rule: recommendations must be preliminary design attention points, not design prescriptions.

# 4. Core implementation logic

The recommendation engine should combine four inputs:

selected province

selected project context

dominant historical driver

priority municipalities / comparable precedents

Recommended formula:

Province + Project context + Dominant driver + Comparable precedents = Project-specific preliminary design focus

| Current Torino example | Meaning for report |
| --- | --- |
| Province: Torino | Use province-level territorial screening language. |
| Project context: Bridge | Generate bridge-specific design focus, not generic territorial advice. |
| Dominant driver: Hydraulic | Focus on hydraulic compatibility, scour susceptibility, flood/debris scenarios and riverbed dynamics. |
| Comparable municipalities: Castellamonte, Ala di Stura, Felleto, Balme, Chialamberto | Use these names in recommendations as historical precedent clusters, not predictions. |

# 5. New section to add to the PDF

Add this section after Interpretation and before Decision Use.

| Section | Recommended content |
| --- | --- |
| Title | Project-Context Design Focus |
| Selected context | Bridge |
| Dominant territorial driver | Hydraulic |
| Suggested paragraph | For bridge interventions in the selected province, ARCUS indicates that preliminary design discussions should prioritise hydraulic compatibility, scour-sensitive configurations, foundation exposure, flood and debris scenarios, and riverbed dynamics. These elements should be verified through site-specific hydraulic, geotechnical and structural studies before design decisions. |

# 6. Project-context recommendation matrix

Use the matrix below to generate context-specific recommendations when the dominant driver is Hydraulic.

| Project context | Preliminary design-support focus |
| --- | --- |
| Bridge | Hydraulic compatibility; scour susceptibility; foundation exposure; riverbed dynamics; flood/debris scenarios; inspection and monitoring accessibility. |
| Road crossing | Flood interruption; drainage capacity; culverts and minor crossings; embankment erosion; road continuity and alternative access. |
| Railway crossing | Track continuity; embankment stability; hydraulic openings; scour near supports; service interruption consequences. |
| Urban infrastructure | Local drainage; underpasses; minor crossings; flood-routing corridors; built-up area interaction; maintenance and civil-protection coordination. |

# 7. Replacement recommendations for Torino / Bridge / Hydraulic

Replace the current generic recommendations with these project-specific recommendations:

For new bridge interventions in the province of Torino, prioritise preliminary hydraulic compatibility checks, including flood levels, flow concentration, debris transport and riverbed evolution.

Treat scour susceptibility as a primary design-screening topic where bridge foundations, abutments or piers interact with active watercourses.

Compare the planned intervention with documented hydraulic total-collapse precedents in Castellamonte, Ala di Stura, Felleto, Balme and Chialamberto before defining investigation priorities.

Use public WMS layers and ARCUS historical precedents as a first-level screening basis, then commission site-specific hydraulic, geotechnical and structural checks before design decisions.

Where historical precedent density is high, include inspection accessibility and monitoring feasibility in the early design discussion.

# 8. Implementation checklist for the platform

| Step | Implementation instruction |
| --- | --- |
| 1 | In Path 01, allow only province selection. Remove or disable custom drawing for this path. |
| 2 | Store projectContext from Step 02 as one of: Bridge, Road crossing, Railway crossing, Urban infrastructure. |
| 3 | Calculate dominantDriver from the selected province historical evidence. Example: Hydraulic. |
| 4 | Build priorityMunicipalities from the top priority events table. |
| 5 | Generate Project-Context Design Focus from projectContext + dominantDriver. |
| 6 | Generate recommendations from the same logic, with fallback generic text only if no matching rule exists. |
| 7 | Render final PDF with controlled engine and without browser headers/footers. |
| 8 | Insert real Leaflet/ARCUS map capture into the Selected Province & Map section. |

# 9. Suggested pseudo-logic

| Condition | Return |
| --- | --- |
| Bridge + Hydraulic | Bridge hydraulic design focus + scour/foundation/flood/debris recommendations. |
| Road crossing + Hydraulic | Road continuity, drainage, culvert and embankment erosion recommendations. |
| Railway crossing + Hydraulic | Track continuity, embankment, hydraulic opening and service interruption recommendations. |
| Urban infrastructure + Hydraulic | Urban drainage, underpasses, minor crossings, flood-routing and civil-protection recommendations. |
| Fallback | General preliminary screening recommendations plus instruction to commission site-specific checks. |

# 10. Final recommended report order

1. Cover

2. 01 Executive Summary

3. 02 Selected Province & Real Map

4. 03 How to Read This Report

5. 04 Key Indicators

6. 05 Hazard Context

7. 06 Historical Evidence

8. 07 Priority Events

9. 08 Comparable Precedents

10. 09 Interpretation

11. 10 Project-Context Design Focus

12. 11 Decision Use

13. 12 Project-Specific Recommendations

14. 13 National Benchmark

15. 14 Data Coverage & Limitations

16. 15 Methodology Snapshot

17. 16 Score and Class Legend

18. A Source Appendix

# 11. Product outcome

With this change, ARCUS Path 01 becomes more than a data report. It becomes a source-backed preliminary design-support briefing. The user selects a province and project context, ARCUS identifies the dominant territorial driver and historical precedents, and the report translates them into safe, useful, project-specific attention points for early design, due diligence and investigation planning.